Engineering Logbook

Wind Tunnel Project

July 10th, 2026

Session 001

Time spent approximately 2 hours

Session Objective: Order all parts and finalize initial design

Progress Made: Initial concept of aluminum honeycomb panel was altered to aramid honeycomb panel. This decision was made due to the lack of workshop tools, and the budget given.

Testing chamber length increases from 6” to 18”. The larger length allows for flow to develop and allows for more room to work with.

4:1 contraction ratio decided with a 12”x12” section allotting more ease to work with.

Parts ordered today:

- Inline Duct Fan

- M3 Screw Kit

- Soldering Iron

- Digital Angle Finder

- Honeycomb

- Arduino

- HX711

- Breadboard Kit

- Acrylic Cutter

- Digital Calipers

- Scissors

- Aluminum Screen

- Stainless Rods

- Weatherstrip

- Load Cell

Questions: How much wood and acrylic should I buy to stay within budget constraints? Should I redesign a lid onto the testing chamber to allow easy access?

Next Session Objective: Finish hand drawings. Solve for contraction ratio and ideal model flow rate

July 15th, 2026

Session 002

Time spent approximately 5 hours

Session Objective: Hand drawings and finalize design

Progress made: All drawings and designs have been finalized. All parts have been purchased and BOM has been finalized.

Design changes include using brackets to fasten the chamber together. Using brackets to connect wood to acrylic. Using 2x3 beam as a wooden post underneath tunnel.

Parts purchased at Home Depot today:

- 2× Anvil 2” heavy-duty spring clamps

- 1× Gorilla Ultimate Epoxy — 0.85 fl oz

- 3× 3-packs neodymium disc magnets — 0.7" × 0.11"

- 1× DAP Kwik Seal Tub & Tile sealant — 10.1 oz

- 2× 4-packs 3½" zinc inside corner braces

- 1× 4-pack 2" zinc inside corner braces

- 4× 12-packs #12 × 2" zinc wood screws

- 2× 12-packs #8 × ¾" zinc wood screws

- 2× 12-packs #10-32 zinc machine screw nuts

- 2× 20-packs #10 zinc bonded sealing washers

- 1× Stanley 15" hand saw

- 2× OPTIX clear acrylic sheets — 0.093" × 20" × 32"

- 2× 2" × 3" × 8’-dimensional lumber boards

- 1× 4' × 8' sheet of ¼" BCX plywood

Questions: Where should I start assembling first? Should I model everything in solidworks first? Should I do cacluations first? What is the next step

Next session Objective: Figure out next step

July 16th, 2026

Session 003

Time spent approximately 3 hours

Session objective: Start cutting panels

Progress made: 4 trapezoidal panels cut out with about an 8th of an inch margin of error. Not an ideal manufacturing set up.

Questions: Will sealant be enough with the corner brackets to keep the tunnel airtight and stable with this margin of error? How will this affect the overall building process?

Next session objective: Cut the 4 12x12” panels needed for the entrance section. Maybe do the diffuser trapezoidal panels depending on time.

July 17th, 2026

Session 004

Time spent approximately 1 hour

Session objective: Cut the needed 4 12x12” panels

Progress made: task completed short day

Questions: N/A after today’s session.

Next session’s objective: Complete trapezoidal cutouts. Hopefully complete all the wooden cutouts other than the fan platform.

July 21st, 2026

Session 005

Time spent approximately 2 hours

Session objective: Cut all acrylic panels

Progress made: All panels are cut slightly larger by an 8th of an inch. Note that edges need to be sanded down.

Questions: how will drilling through this acrylic work? Will it crack? How would I work around this if it is an issue?

Next session objective: finish the trapezoidal panels and 12x12” panels.

July 23rd, 2026

Session 006

Time spent approximately 2.5 hours

Session objective: Finish cutting trapezoidal and 12x12” panels

Progress made: all panels have been cut out. All panels have been measured and geometrically selected in order to optimize fitting (panels have about a +-0.125” error). Troubleshot the inconsistencies and have a plan to deal with gaps. (Run sealant along inside edges and smooth it out into a filet shape.)

Questions: Will I need to recut any panels? Will the sealant be enough to keep it airtight? How will I mount the fan? How will the load cell mounting work?

Next session objective: Assemble the contraction and diffuser sections fully. Ambitiously and ideally, also assemble the inlet section as well.

July 27th, 2026

Session 008

Time spent approximately 6 hours

Session Objective: Assemble diffuser and contraction section fully

Progress made: diffuser section fully assembled and sealed off with about a quarter inch error (shorter side) which is more ideal than an overshoot. The contraction section has been assembled, however not sealed and needs to be fixed. The geometry of it is rhombus like and not square like for the 6x6” section. Need to reassemble and adjust contraction section.

Questions: Will I be able to maintain airtightness with a quarter inch error? Will the contraction section be fixed after reassembly? Will I be able to successfully connect the acrylic section?

Next session objective: finish contraction section, inlet section, and acrylic section. That way we can start connecting the sections, build the supports and begin testing.

August 6th, 2026

Session 009

Time spent approximately 3 hours

Session objective: fix and seal off diffuser section

Progress made: Realigned diffuser section and sealed it off completing it entirely. Started inlet section only put the honeycomb on and screwed 2 panels.

DESIGN CHANGE MADE, fan will push air instead of pull air inside. Design decision made due to lack of tools to cut a circular hole into wood for the fan (jigsaw which I do not have). Due top constraints design has been changed.

Questions: Will everything align and seal just fine?

Next session objectives: build and fully finish inlet section.

August 11th, 2026

Session 010

Time spent approximately 2 hours

Session objective: Build inlet section fully and seal it off

Progress made: Inlet fully built, sealed off, and aluminum sheets attached

Not much to say for today.

Questions: N/A

Next session objective: Build acrylic section